The following is additional information regarding Best and Final (BAFO) to the Request for Proposal #3681 titled The Center City Connector Streetcar Vehicle released on 02/10/2017.  The due date and time for the BAFO responses is 07/11/2017, 4:00 PM (Pacific).  This addendum includes both questions from prospective proposers and the City’s answers, and revisions to the BAFO/RFP. This addendum is hereby made part of the RFP and therefore, the information contained herein shall be taken into consideration when preparing and submitting a BAFO.

| Item # | Date Received | Date Answered | Vendor’s Question | City’s Answer | RFP Revisions (Red text is modification to existing language) |
| --- | --- | --- | --- | --- | --- |
| 1 | n/a | 6/16/17 | n/a |  | Reference Rev 6/15/17, Special Provisions when submitting your BAFO |
| 2 | n/a | 6/16/17 | n/a |  | Use Rev 2, Price Summary Form when submitting BAFO pricing |
| 3 | n/a | 6/16/17 | n/a |  | Due to the expected lead time of the initial delivery, please submit your proposal with minimum 70% Buy America compliancy. The following is added to Technical Proposal Response Form (Revised 2/22/17) Sub-part F: Ability to meet Buy America Requirements. Regardless of delivery date, the City requires that the cost of components and subcomponents produced in the United States must be 70% or more of the cost of all components of the rolling stock. |
| 4 | n/a | 6/16/17 | n/a |  | SDOT has an estimated budget range of $48 - $52M including spare parts . |
| 5 | n/a | 6/16/17 | n/a |  | Reference Exhibit B, Contract Terms & Conditions, Section 46 Indemnification. To the extent permitted by law, the Vendor shall protect, defend, indemnify and hold the City harmless from and against all claims, demands, damages, costs, actions and causes of actions, liabilities, fines, penalties, judgments, expenses and attorney fees, resulting from the injury or death of any person or the damage to or destruction of property, or the infringement of any patent, copyright, or trademark, or trade secret caused by the fault of vendor, its sub-contractors, assigns or other persons performing work on the project at vendor’s request, or their violation of any law, ordinance or regulation, contract provision or term, or condition of regulatory authorization or permit, except for damages resulting from the sole negligence of the City. As to the City of Seattle, the Vendor waives any immunity it may have under RCW Title 51 or any other Worker’s Compensation statute. Notwithstanding anything herein to the contrary, Contractor’s liability to The City arising out of, or in conjunction with, this Contract shall not exceed the then-current total contract price, provided that this limitation shall not apply to any obligation of Contractor to indemnify The City based on claims of third parties (including employees of The City and Contractor, or to Contractor’s liability resulting from its gross negligence or willful misconduct. This indemnity shall survive the termination of this Contract or final payment hereunder. This indemnity is in addition to any other rights or remedies which The City and the other parties to be indemnified may have under the law or under this Contract. |
| 6 | 6/16/17 | 6/23/17 | According to your Letter dated in June 9, 2017 Exceptions to the City’s Terms and Conditions, Exhibit B (Rev 1) may be submitted by Vendor. Could SDOT confirm that Exceptions to the City’s Special Provisions, Exhibit C (Rev 1) may also be submitted by Vendor? Please confirm this is correct. | Proposers must accept all T&Cs (and SPs) as currently written. Any changes during negotiations are at the City’s sole discretion, and proposed exceptions should not be construed to be a Condition of a Proposal. |  |
| 7 | 6/16/17 | 6/16/17 | When will SDOT issue the addendum with the new Price Summary (Form)? | See Item #2 above |  |
| 8 | 6/16/17 | 6/23/17 | Exhibit C (Rev 1), Progress Payments Reference is made to article 7.1 payment milestone E. There seems to be a typo at the end of the milestone description. Please replace “car” with “car shell”. |  | Milestone Payment Table in Section 7.1 of the SPs: Revise Milestone E to read: “Ten percent (10%) of the unit price for each streetcar upon completion of the car shell (10 times)” |
| 9 | 6/16/17 | 6/23/17 | Technical Specs, Section 2.8.2, Pure tones Is it acceptable that pure tone noise will be considered significant in this context if any one-third octave band sound pressure level is 5 dB, analysed between 500Hz and 4000Hz frequency 1/3-octave bands, or more, higher than the arithmetic average of the two adjacent bands containing no pure tones? | Acceptable. See updated Technical Specifications, Rev 2.0 | Updated Technical Specifications 2.0 |
| 10 | 6/16/17 | 6/23/17 | Technical Specs, Section 4.3.4, Draft Gear and Anchorage Taking into account that the coupler is only for rescue purposes, it is acceptable that the coupler did not have to absorb energy coupling since it is designed to be coupled manually. In addition, the vertical support is also not necessary for this same reason. Please confirm this is correct. | No change to Technical Specification |  |
| 11 | 6/16/17 | 6/23/17 | Technical Specs, Section 5.7.2, Interior View Display Screen Is it acceptable that alternatively a mirror could be used for this purpose, instead of using a dedicated LCD screen? |  | Acceptable. See updated Technical Specifications, Rev 2.0, See Item #9 above |
| 12 | 6/16/17 | 6/23/17 | Technical Specs, Section 7.3.1 HVAC Units Is it acceptable to use 3 HVAC units for the passenger compartment without installing one HVAC unit in each vehicle body section, since all the systems are interconnected through a common air distribution duct? | Alternative HVAC arrangements may be proposed for review and approval by the Owner. |  |
| 13 | 6/16/17 | 6/23/17 | Technical Specs, Section 13.4.11.2 Information Signs LCD screens are required for the passenger compartment. Inclusion of these screens will not allow compliance with the interior ceiling height (including anything being mounted on it). We therefore request the use of interior LED destination displays be allowed instead of LCD screens. | The LCD screens for the information signs will not be considered when calculating ceiling height. |  |
| 14 | 6/16/17 | 6/23/17 | Technical Specs, Section 13.7.2 Cameras Is it acceptable to use front cameras which provide 1280x800 pixel resolution? |  | Acceptable. See updated Technical Specifications, Rev 2.0, See Item #9 above |
| 15 | 6/16/17 | 6/23/17 | Technical Specs, Section 13.7.8.6 Storage Capacity Please confirm that provision of storage capacity of 15 days of surveillance video instead of 21 days is sufficient. |  | Acceptable. See updated Technical Specifications, Rev 2.0, See Item #9 above |
| 16 | 6/16/17 | 6/23/17 | Technical Specs, Section 13.9.3 Vehicle Status and Health Data Can the City please clarify the reason for inclusion of such a requirement? Please confirm that the proposal to provide a software tool to enable the user to access vehicle status and health data logged in the MDS is correct. |  | Acceptable. See updated Technical Specifications, Rev 2.0, See Item #9 above |
| 17 | 6/16/17 | 6/23/17 | Technical Specs, Section 13.10.4 Event Recorder Construction Requirements Is it an acceptable alternative to provide a US service proven Event Recorder which is not constructed in accordance with the requirements of IEEE 1482.1, but which includes a Crash Protected Memory (CPM) that is fully compliant with the standard IEEE 1482.1? | Per TS 13.10.4 Crash Protected memory (CPM) is compliant per item 1. |  |
| 18 | 6/16/17 | 6/23/17 | Contract T& C’s Exhibit B. Rev 1 BAFO Page 15 Will SOOT please set the language for Limitation of Liability apart in an individuated provision of the document? | Proposers must accept all T&Cs (and SPs) as currently written. Any changes during negotiations are at the City’s sole discretion, and proposed exceptions should not be construed to be a Condition of a Proposal. |  |
| 19 | 6/16/17 | 6/23/17 | Technical Specifications, Section 2 Will the City please clarify the duration of the demonstration period? | The demonstration period begins at Conditional Acceptance of the fifth car and ends one year after Conditional Acceptance of the last car. |  |
| 20 | 6/16/17 | 6/23/17 | Exhibit C, Special Provisions, Section 13, Warranty Will the City add language to transfer the warranty burden from the Surety to the Contractor for components and parts warrantied beyond 2 years? | No change to requirements. |  |
| 21 | 6/16/17 | 6/23/17 | Technical Specifications, Section 2 Page 21 Some of the contractor’s subsystems have been tested to EN61000-4-4 and EN 61000-4-5. Most subsystems have not been tested to 47 CFR Part 15 and ANSI C63.12, but have been tested toEN5012-3-2. Will this be accepted as acceptable, equivalent testing by the City? | No change to Technical Specification, this is a design review question; all equipment installed on the vehicle shall be compliant with Title 47 CFR 15 and 90. During design review alternative testing and standards maybe proposed to show compliance with Title 47 CFR 15 and 90. |  |
| 22 | 6/16/17 | 6/23/17 | Technical Specifications, Section 2 Page 23 The Contractor conforms to MIL-STD-8828 for System Safety. Will this be accepted as an equivalent alternative? | No, DOT-FTA-MA- 26-5005-00-01 is the FTA guideline for performing Hazard Analysis on Transit Projects, which references MIL-STD-882B, the format from DOT-FTA-MA-26-5005-00-01 shall be followed. |  |
| 23 | 6/16/17 | 6/23/17 | Technical Specifications, Section 2 Pages 7 & 8 Will the City accept a pantograph lockdown height of 156 inches? | No, No change to Technical Specification |  |
| 24 | 6/16/17 | 6/23/17 | Technical Specifications, Section 3 Will the City accept skirts that are not hinged? | No, No change to Technical Specification |  |
| 25 | 6/16/17 | 6/23/17 | Technical Specifications, Section 5 Will the City accept a speedometer with a 4 inch diameter? | No, No change to Technical Specification |  |
| 26 | 6/16/17 | 6/23/17 | Technical Specifications, Section 6 Will the City accept a ramp without a wear strip? | No, No change to Technical Specification |  |
| 27 | 6/16/17 | 6/23/17 | Technical Specifications, Section 6 Will the City accept derlin threshold plate rollers that require periodic lubrication for the front flap rubber and torsion spring? | No, No change to Technical Specification |  |
| 28 | 6/16/17 | 6/23/17 | Technical Specifications, Section 6 Will the City accept a ramp deployment speed of 100mm/s? | No, No change to Technical Specification |  |
| 29 | 6/16/17 | 6/23/17 | Technical Specifications, Section 6 Will the City accept a peak threshold adjustment of 300N? | No, No change to Technical Specification |  |
| 30 | 6/16/17 | 6/23/17 | Technical Specifications, Section 9 Will the city please waive 1a and eliminate the 1 b requirement for the capability of to operate without speed restriction? | No, No change to Technical Specification |  |
| 31 | 6/16/17 | 6/23/17 | Technical Specifications, Section 9 What is the highest possible acceptable resonant frequency that would be acceptable to the City? | No, No change to Technical Specification |  |
| 32 | 6/16/17 | 6/23/17 | Technical Specifications, Section 9 Wilt the City please waive/remove 9.9.3? | No, No change to Technical Specification |  |
| 33 | 6/16/17 | 6/23/17 | Technical Specifications, Section 9 Will the City please remove the requirement prohibiting electrolytic capacitors? | No, No change to Technical Specification |  |
| 34 | 6/16/17 | 6/23/17 | Technical Specifications, Section 9 Will city please waive the requirement for derating by a factor of four? | No, No change to Technical Specification |  |
| 35 | 6/16/17 | 6/23/17 | Technical Specifications, Section 9 Will the City please remove the requirement for Arc-fed blowout coils for AC contactors? Please also restrict the 5 million cycle rating to contactors to which the requirement applies | No, No change to Technical Specification |  |
| 36 | 6/16/17 | 6/23/17 | Technical Specifications, Section 10 Will the City please remove the requirement to identify the potential for excess EMI? | No, No change to Technical Specification |  |
| 37 | 6/16/17 | 6/23/17 | Technical Specifications, Section 10 Will the City remove the requirement for automatic filter selftest initiation? | No, No change to Technical Specification |  |
| 38 | 6/16/17 | 6/23/17 | Technical Specifications, Section 10 Will the City remove the requirement for precharge circuit implementation? | No, No change to Technical Specification |  |
| 39 | 6/16/17 | 6/23/17 | Technical Specifications, Section 12 Will the City permit the use of a pressure switch for the auxiliary release unit? | No, No change to Technical Specification |  |
| 40 | 6/16/17 | 6/23/17 | Technical Specifications, Section 13 Will the City waive requirement list item No. 2 (provide satellite prediction)? | No, No change to Technical Specification |  |
| 41 | 6/16/17 | 6/23/17 | Technical Specifications, Section 13 Wil the City accept equipment that complies with the EN 50155 standard? | No, No change to Technical Specification |  |
| 42 | 6/21/17 | 6/23/17 | n/a |  | We are no longer requiring a 1-day site visits to each Proposer’s facility as part of this BAFO process. The City reserves the right to visit the facility of only the successful Proposer. This site visit will remain as a pass/fail. |